# 🎧 Tour Audio Solution — Tour Director Cheat Sheet

Quick Reference & Troubleshooting Guide | EF Tour Audio Solution | Confidential

_________________________________________________________________

## 📦 What's in Your Kit

| Item | Purpose |
| --- | --- |
| GL.iNet WiFi Router | Broadcasts audio to all travelers |
| 20,000mAh Battery Pack | Powers the router for 8+ hours |
| USB Cable | Connects router to battery pack |
| Sling Bag | Carry everything hands-free |
| QR Code Card | Travelers scan to connect instantly |

**Before every tour:** Make sure the battery pack is fully charged and the router powers on.

_________________________________________________________________

## 🟢 PART 1 — Tour Guide Setup (Broadcasting Audio)

**Note for Tour Directors:** The *tour guide* (who may or may not be the same person as the Tour Director) is the one who will broadcast audio to the group. The Tour Director's role is to get everyone connected, help troubleshoot, and assist the guide with their setup if needed. Both the TD and the guide will need to connect their phones — just to different networks (see below).

_________________________________________________________________

### Step 1 — Power On the Router

- Connect the router to the battery pack using the USB cable.

- Wait about **60 seconds** for the router to fully boot up.

- The router's LED light will turn solid (not blinking) when it is ready.

_________________________________________________________________

### Step 2 — Connect the Guide's Phone to the Guide Network

The guide connects to a separate, dedicated 5 GHz network for better performance when broadcasting.

| Network Name | Password |
| --- | --- |
| EF-TOUR-GUIDE | EFLearnFun |

- Open **Wi-Fi Settings** on the guide's phone.

- Find **`EF-TOUR-GUIDE`** and connect using the password above.

- Your phone may show **"No Internet Connection"** — this is **normal and expected**.

_________________________________________________________________

### Step 3 — Open the Tour Guide App

- Open your phone's browser (Safari on iPhone, Chrome on Android).

- Go to: **`audio.eftours.travel`**

- If you see a security warning ("Your connection is not private"), tap **Advanced → Proceed** to continue. This is expected and safe.

- Select **"Tour Guide"** from the app menu.

- Tap **"Start Broadcasting"** — you are now live.

_________________________________________________________________

### 🎙️ Audio Optimization Tips for the Guide

- Hold the router high. Clip it to your bag strap or hold it at shoulder height. This gives the signal a clear line of sight over the group and improves range significantly.

- Turn on Voice Isolation. On iPhone: while on the app, swipe down to open Control Center → tap the microphone icon → select "Voice Isolation." This filters out wind, crowd noise, and echo — making your voice much clearer.

- Speak toward your phone's microphone. Hold your phone close to your mouth (like a walkie-talkie), not out in front of you.

- Stay within the group. The router signal reaches about 30–50 meters in open air. If you walk too far ahead, travelers at the back may drop off.

- Avoid putting the router in your bag or pocket. Fabric and your body block the signal. Keep it exposed and elevated.

_________________________________________________________________

### 📊 Guide App Features

The guide app has several built-in tools to help you manage the tour experience:

- Performance Stats — The guide app shows a live dashboard with the number of connected travelers, signal quality, and audio stream health. Check this at the start of each stop to confirm everyone is connected.

- Live Closed Captions — The app can transcribe your speech to text in real time and broadcast it as subtitles to all traveler screens. To turn this on, tap the CC button in the guide app. No internet is required — it runs entirely on your phone. This is especially helpful for travelers who are hard of hearing.

- Traveler Reactions — Travelers can send emoji reactions (👍 👎 ❤️ 😂 🤯) that appear on your guide screen in real time. You can use these for polls and quiz games — for example, ask a true/false question and have travelers respond with 👍 for True and 👎 for False. Great for keeping the group engaged on the bus!

- Lock Screen / Power Save Mode — If you need to save your phone's battery during a long tour, tap the lock icon in the app. The screen dims to near-black but audio continues broadcasting uninterrupted.

_________________________________________________________________

## 🔵 PART 2 — Traveler Connection Instructions

Get travelers connected on the bus ride over — before you arrive at the first stop. This saves time and lets you troubleshoot any issues before the tour begins. You can even run a fun true/false quiz game using the 👍 and 👎 reaction buttons as a warm-up!

_________________________________________________________________

### Traveler Network

| Network Name | Password |
| --- | --- |
| EF-TOUR-001 | EFLearnFun |

Travelers connect to the 2.4 GHz traveler network — this band has better range and penetrates through walls and crowds more effectively.

_________________________________________________________________

### How to Connect (2 Ways)

Option A — Scan the QR Code (Easiest)

| - Open your phone's camera app. - Point it at the QR code on the card your Tour Director is holding. - Tap the notification that appears — your phone connects automatically. - Open your browser and go to **`audio.eftours.travel`**. | 1. Wifi QR Code | 2. App QR Code |
| --- | --- | --- |

Option B — Connect Manually

- Open **Wi-Fi Settings** on your phone.

- Find the network **`EF-TOUR-001`** and enter the password: **`EFLearnFun`**

- Open your browser and go to **`audio.eftours.travel`**.

_________________________________________________________________

### 💡 Tips for Travelers

- Plug in your earbuds for the best audio experience.

- Turn off Silent Mode if you are listening through your phone's built-in speaker. Silent mode will prevent audio from playing through the speaker. (If you are using earbuds or headphones, this does not affect you.)

- Keep the app open on your screen. If you switch to another app or lock your screen, audio may pause. Use the in-app camera to take photos without leaving the audio stream. (Note: the in-app camera is only available in the full browser app at audio.eftours.travel — it will not appear in the captive portal pop-up browser.)

- You will see "No Internet" on your phone — this is fine. The audio runs on the local tour network, not the internet.

- There is a 1–2 second audio delay — this is normal and by design. Don't worry if the guide's voice sounds slightly behind what you see.

- Low battery? Tap the lock icon in the app to enter Lock Screen / Listening Mode — it dims your screen to save battery while keeping audio running. Your battery level is shown in the app.

_________________________________________________________________

### 🌟 Traveler App Features

- Live Closed Captions (CC) — If the guide has turned on captions, you will see a subtitle bar at the bottom of your screen showing what the guide is saying in real time. Helpful if you have difficulty hearing, or just want to follow along in text.

- Emoji Reactions — Tap the reaction buttons (👍 👎 ❤️ 😂 🤯) to respond to the guide. Your reaction is visible to everyone in the group. The guide may use these for polls or quiz questions — just tap the emoji that matches your answer!

- Lock Screen / Listening Mode — Tap the lock icon to dim your screen and save battery. Audio keeps playing. Tap anywhere to wake the screen back up.

- Battery Indicator — Your phone's battery level is shown in the app so you can keep an eye on it without leaving the stream.

_________________________________________________________________

## 🔴 PART 3 — Troubleshooting Connection Issues

### ❌ Problem: iPhone won't load audio.eftours.travel in Safari

Most likely cause: iCloud Private Relay is turned on.

Private Relay routes your internet traffic through Apple's servers, which blocks access to the local tour network.

Fix Option 1 — Use the Captive Portal Browser (Easiest)

- When you first join the Wi-Fi network, a pop-up browser window should appear automatically. Use that window to open the app. Private Relay is automatically suspended in this browser.

- ⚠️ Note: The in-app camera is not available in the captive portal browser. If you want to use the camera feature, use Fix Option 2 or 3 below to open the full browser instead.

Fix Option 2 — Use the IP Address Directly

- In Safari, type: 192.168.8.1 (instead of audio.eftours.travel) and press Go.

Fix Option 3 — Temporarily Disable Private Relay

- Open **Settings → [Your Name] → iCloud → Private Relay**

- Toggle **Private Relay OFF**

  - You can choose "Turn Off For Today" — Private Relay will automatically turn itself back on tomorrow. You do not need to remember to re-enable it.

  - Or toggle it fully off and turn it back on manually after the tour.

- Open Safari and go to `audio.eftours.travel`.

_________________________________________________________________

### ❌ Problem: Phone keeps auto-connecting to a different network

Some phones automatically switch to a nearby network that has internet, even if you connected to the tour network.

Fix:

- Go to **Wi-Fi Settings** and tap the tour network name.

- On iPhone: turn off **"Auto-Join"** for any other nearby networks.

- On Android: tap the tour network → select **"Forget"** for competing networks, or tap **"Stay Connected"** when the phone asks if you want to switch.

- If the phone keeps switching, turn on **Airplane Mode**, then turn Wi-Fi back on and connect to the tour network only.

_________________________________________________________________

### ❌ Problem: "No Internet Connection" warning on the phone

This is expected — do not worry. The tour audio system runs on a private local network. It does not need internet to stream audio. Travelers can safely ignore this warning and proceed to audio.eftours.travel.

- On iPhone, you may see a small warning icon on the Wi-Fi symbol. This is normal.

- On Android, you may get a pop-up asking if you want to stay connected. Tap "Yes, stay connected."

_________________________________________________________________

### ❌ Problem: Audio is cutting out or dropping

Try these steps in order:

- **Check your distance.** Move closer to the guide. If you are at the back of a large group, signal may be weak.

- **Check the router position.** Ask the Tour Director to raise the router higher (shoulder height or above).

- **Refresh the page.** Close and reopen `audio.eftours.travel` in your browser.

- **Reconnect to the Wi-Fi network.** Go to Settings → Wi-Fi → tap `EF-TOUR-001` to reconnect.

- **Restart your browser.** Close all browser tabs and reopen `audio.eftours.travel`.

_________________________________________________________________

### ❌ Problem: Audio paused when I switched apps or took a photo

The browser cannot keep audio running in the background on most phones — this is a phone limitation, not a bug.

Fix:

- Use the in-app camera (there is a camera button inside the app at audio.eftours.travel) to take photos without leaving the audio stream.

- The in-app camera is only available in the full browser — not in the captive portal pop-up window.

- If you already left the app, just go back to your browser and go to audio.eftours.travel — audio will resume.

_________________________________________________________________

### ❌ Problem: The captive portal pop-up didn't appear

Some phones don't automatically show the captive portal when joining a new network.

Fix:

- Open your browser manually and type: audio.eftours.travel or 192.168.8.1

- The app should load directly.

_________________________________________________________________

### ❌ Problem: No audio through the phone speaker

Check that Silent Mode is off. On iPhone, the physical switch on the left side of the phone should not show an orange stripe. If it does, flip it to unmute.

- This only affects the built-in speaker. If you are using earbuds or headphones, silent mode does not matter.

_________________________________________________________________

### ❌ Problem: Traveler has no phone, dead battery, or no earbuds

- No phone: Ask them to stand close to the guide and listen directly.

- Dead battery: Remind travelers the night before to charge their phones. Carry a small portable battery pack to lend if needed.

- No earbuds: The audio plays through the phone speaker — they can still hear, just less clearly. Make sure silent mode is off. Ask them to stand near the front of the group.

_________________________________________________________________

## ⚙️ PART 4 — Tips & Best Practices

### Before the Tour

- ✅ Charge the battery pack fully the night before.

- ✅ Power on the router and confirm it boots up (solid LED light).

- ✅ Test the connection on your own phone before the group arrives.

- ✅ Have the QR code card ready and visible.

- ✅ Remind travelers to charge their phones overnight.

- ✅ Help the guide connect to EF-TOUR-GUIDE and open the guide app before departure.

### On the Bus — Connect Early

- ✅ Get travelers connected on the bus ride over — before you arrive at the first stop. This gives you time to troubleshoot any issues without holding up the tour.

- ✅ Once everyone is connected, run a quick warm-up quiz using the reaction buttons. Ask a true/false question about the destination and have travelers tap 👍 for True and 👎 for False. It's a fun way to build excitement and confirm the system is working for everyone.

### During the Tour

- ✅ Keep the router elevated — clip it to the guide's bag strap at shoulder height.

- ✅ Enable Voice Isolation on the guide's iPhone before broadcasting.

- ✅ Keep the group reasonably close together (within ~30–50 meters).

- ✅ If you are in a dense crowd or museum, move the router to the highest point possible.

- ✅ Check the guide app's performance stats at each stop to confirm everyone is still connected.

### Multiple Groups in the Same Area

- Each router creates its own separate network — groups will not interfere with each other's audio.

- However, in very crowded areas (e.g., a busy plaza with many tour groups), Wi-Fi congestion can increase. If audio quality drops, try switching to a less-used Wi-Fi channel in the router admin panel.

### What the Captive Portal Can (and Cannot) Do

- ✅ Can do: Automatically suspend iCloud Private Relay on iPhones, provide a simple browser for travelers to open the app, and serve as a fallback connection method.

- ❌ Cannot do: Provide real internet access, load external websites, show the in-app camera feature, or work as a general-purpose hotspot. It is a local-only connection for the tour audio system only.

- 💡 Best practice: Encourage travelers to open the full browser at audio.eftours.travel rather than staying in the captive portal, so they have access to all features including the in-app camera.

_________________________________________________________________

## 🔐 PART 5 — Refreshing the Security Certificate

The SSL certificate for audio.eftours.travel expires approximately every 90 days. Renewal takes about 60 seconds and requires a temporary internet connection.

**What is a certificate?** It is a small file on the router that tells browsers the connection is secure. When it expires, travelers will see a security warning they cannot get past. Renew it before this happens.

### When to Renew

- The admin portal shows the number of days remaining on the certificate — check this periodically.

- Browsers will show a persistent security warning if the certificate has already expired.

- Renew proactively before the start of a new tour season or if you are notified by your coordinator.

_________________________________________________________________

### How to Renew — Via the Admin Portal (Recommended)

Step 1 — Connect the Router to the Internet

- Connect the router to a hotel Wi-Fi or your phone's hotspot using the "Connect to WiFi" option in the router admin panel.

Step 2 — Open the Admin Portal

- On a device connected to the tour Wi-Fi, open a browser.

- Go to: https://audio.eftours.travel/admin?token=EFAdminAudio02141

- You are now in the admin portal.

Step 3 — Run the Update

- Tap **"Update Device Now."**

- Watch the progress steps — the portal will automatically fetch, validate, back up, and install the new certificate.

- The server restarts itself. Travelers will reconnect within a few seconds.

Step 4 — Verify It Worked

- Connect your phone to the tour Wi-Fi (`EF-TOUR-001`).

- Open Safari or Chrome and go to `audio.eftours.travel`.

- The app should load **without** a security warning. If the warning is gone, you are all set.

**If the renewal fails:** Contact your coordinator. Do not run a tour with an expired certificate — travelers will see a security warning that may prevent them from connecting.

_________________________________________________________________

## 📞 Quick Reference Card

| Situation | What to Do |
| --- | --- |
| iPhone won't load the app | Use captive portal browser, or disable Private Relay in Settings |
| "No Internet" warning | Normal — ignore it and proceed to audio.eftours.travel |
| Phone switched to another network | Manually reconnect to EF-TOUR-001; tap "Stay Connected" |
| Audio cutting out | Move closer to guide; refresh the page; reconnect to Wi-Fi |
| Audio paused after switching apps | Return to browser; use in-app camera next time |
| No audio through speaker | Turn off Silent Mode on the phone |
| Captive portal didn't appear | Type audio.eftours.travel or 192.168.8.1 directly in browser |
| In-app camera not showing | Open full browser at audio.eftours.travel (not captive portal) |
| Security warning won't go away | Certificate may need renewal — see Part 5 |
| Traveler has no phone | Stand near the guide and listen directly |

_________________________________________________________________

EF Tour Audio Solution | Tour Director Reference Guide | Confidential Questions? Contact Jeff Duffield — jeff.duffield@ef.com